Background: Safely Riding a Car.

You are tasked to design a combinational logic circuit that operates an alarm in a car

whenever the driver and/or passenger seats are occupied, and the seatbelts are not fastened

when the car is started. The active-HIGH signals DRIV and PASS indicate the presence of the

driver and passenger, respectively, and are taken from pressure-actuated switches in the

seats. The signal IGN is active-HIGH when the ignition switch is on. The signal 𝐵𝐸𝐿𝑇𝐷̅̅̅̅̅̅̅̅̅̅ is active-

LOW and indicates that the driver’s seatbelt is unfastened. 𝐵𝐸𝐿𝑇𝑃̅̅̅̅̅̅̅̅̅ is the corresponding signal

for the passenger seatbelt.

The alarm will be activated (LOW) whenever the car is started and either of the front seats is

occupied and its seatbelt is not fastened.

Your goal is to design the logic circuit needed to fulfill the above conditions. You must use the

problem-solving process (PSP) taught during the lectures alongside the Boolean design

process (BDP). Below is a step-by-step guide to assist you in this endeavour. Your tutors will

also assist you in this.

Step 1: Understand and Define the Problem (Analyse)

Step 2: Organise and Describe the Data

Step 3: Plan the Solution (Design the Algorithm)

3.1 Algorithm (in plain English)

3.2 Truth Table

3.3 Boolean Expressions (SOP)

3.4 Flowchart

3.5 Pseudocode

Step 4: Test and Refine the Solution (Debug and Verify)

- This problem is related to designing a combinational logic circuit for a car that operates an alarm whenever the driver and/ or passenger seats are occupied, and the seat belts are not fastened for safety purposes. The alarm for not fastened seat belts rings when the car is turned on (IGN), one of the seats are occupied either driver or passenger (DRIV/PASS), and the seat belt is not fastened for any seat (BELTD). If the seat belt is fastened/unfastened it will send a signal (BELTP).

- Inputs: IGN – Ignition (to turn on or off the car)
DRIV – Driver (1 = Driver is present. 0= not present)
PASS- Passenger (1 = passenger is present. 0= not present)

BELTD – Seat belt (1= fastened, 0= unfastened)

BELTP- Signal for seat belt (1= seat is fastened, 0= not fastened)

IMP: Sensor is active LOW for BELTD and BELTP

Output: ALARM= 0 (not fastened so alarm rings,) 1 (fastened, alarm doesn’t ring)
IMP: Sensor is active LOW for ALARM

INPUTS

| Symbols | Function | Meaning |
| --- | --- | --- |
| IGN | 1= Car is turned on (HIGH) 0= Car is turned off (LOW) | Turns on/off the car |
| DRIV | 1= Driver seat is active (HIGH) 0= Driver seat is not active (LOW) | If the driver seat is occupied or not |
| PASS | 1= Passenger seat is active (HIGH) 0= Passenger seat is not active (LOW) | If the passenger seat is occupied or not |
| BELTD | 1= Seat belt is fastened (HIGH) 0= Seat belt is unfastened (LOW) | It is the signal given if the seat belt is fastened or not for driver |
| BELTP | 1= Seat belt is fastened (HIGH) 0= Seat belt is unfastened (LOW) | It is the signal given if the seat belt is fastened or not for passenger |

OUTPUT

| ALARM | 1= alarm is off seat belt is fastened (HIGH) 0= alarm rings seat belt is unfastened (LOW) | Alarm rings when not fastened |
| --- | --- | --- |

- > Monitor the car ignition = Must be HIGH

- Driver is seated and the seatbelt is not fastened = DRIV = 1 and BELTD’ = 0

- Passenger is seated and the seatbelt is not fastened = PASS = 1 and BELTP’ = 0

- If either condition of above is TRUE then ALARM is turned on.

- No conditions met, the ALARM is turned off.

- Truth Tables

| DRIV | PASS | BELTD’ | BELTP’ | IGN | ALARM’ |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

- Boolean expressions (SOP)

DRIV. PASS. BELTD. BELTP. IGN

DRIV. PASS. BELTD. BELTP. IGN

DRIV.PASS. BELTD. BELTP. IGN

DRIV. PASS. BELTD. BELTP. IGN

DRIV.PASS.BELTD. BELTP. IGN

ALARM=DRIV. PASS. BELTD. BELTP. IGN+DRIV. PASS. BELTD. BELTP. IGN+DRIV.PASS. BELTD. BELTP. IGN+DRIV. PASS. BELTD. BELTP. IGN+DRIV.PASS.BELTD. BELTP. IGN

- PSUEDOCODE

IF (DRIV. PASS. BELTD. BELTP. IGN) OR

(DRIV. PASS. BELTD. BELTP. IGN) OR

(DRIV.PASS. BELTD. BELTP. IGN) OR

(DRIV. PASS. BELTD. BELTP. IGN) OR

(DRIV.PASS.BELTD. BELTP. IGN) OR

Then: 
ALARM = 0

Else:

ALARM = 1